Renewable natural gas production via ex-situ biomethanation in a novel hollow-fiber membrane biofilm reactor

Heather Nielsen1, Thomas Lippert1, Pranava Nannapuraju2, Andrew Freiburger2, Bofan Wang1, Daniel Morrow1, Gianna Brogley1, Rodolfo Amezcua-Shoeman2, Yupo Lin3, Lutgarde Raskin3, Keith Tyo2, George F. Wells1*

1 Northwestern University, Department of Civil & Environmental Engineering, Evanston, IL 60208, USA

2Northwestern University, Department of Chemical Engineering, Evanston, IL 60208, USA

3Argonne National Laboratory, Applied Materials Division, Lemont, IL 60439, USA

4 Yale University, Department of Chemical & Environmental Engineering, New Haven, CT 06511, USA

*Corresponding author: george.wells@northwestern.edu

# Abstract

Ex-situ biomethanation processes are an emerging option for renewable natural gas (RNG) production from hydrogen (H2) and carbon dioxide (CO2), but suffer from the low solubility and consequent poor gas-liquid mass transfer characteristics of H2. Membrane biofilm reactors (MBfR) offer a route to overcome H2 mass transfer limitations through bubbleless diffusion. In the limited prior work in this area, however, MBfRs have underperformed, ostensibly due to low surface-to-volume ratios (SVR). This work tested an MBfR with a high-SVR (829 m2/m3) under mesophilic operation for 331 days. The MBfR achieved a specific CH4 production rate of 6.18 ± 0.11 m3CH4/m3d, while regularly yielding gas quality >98% CH4. Conversely, This was achieved with a relatively low membrane-normalized production rate (7.5 m3CH4/m2d), suggesting opportunities for further performance improvements via optimization of the spatial arrangement of the membranes. Microbial community analyses via partial 16S rRNA gene sequencing demonstrated the emergent biofilm was dominated by three amplicon sequence variants (ASVs) in the family Methanobacteriaceae (80.2% summed relative abundance by day 300). One of these three putative hydrogenotrophic methanogens, classified only at the genus level Methanobacterium, exhibited interspecific competition with the other two and was most abundant ASV under stable substrate loadings. Methanobacterium midas_s_9526, however, exhibited a strong, statistically significant correlation with increased H2 loadings and was the most abundant ASV by day 300. Flanking non-methanogenic community members appear to be unique to each methanogenic ASV, suggesting specificity in relationships between different methanogens and their flanking communities. Overall, these results demonstrate that high-SVR MBfRs can support robust high-rate, high quality RNG production and reveal new design considerations required to fully leverage membrane-based gas delivery.

# Structured Abstract

To be filled later.

# Keywords (<7)

Ex-situ biomethanation, biomethane, renewable natural gas, membrane biofilm reactor, hollow fiber membranes, methanogen, anaerobic microbiome

# Highlights

- 331-day test of a novel, high-SVR (829 m2/m3) MBfR for ex-situ biomethanation

- Maximum specific CH4 production rate, 6.18 ± 0.11 m3CH4/m3d

- High quality RNG production, 95.7 ± 1.7% CH4, 1.0 ± 0.4% CO2, and 3.2 ± 1.6% H2

- Substrate conversion efficiencies were up to 88.8 ± 2.6% for H2 and 88.0 ± 2.3% for CO2

- Hydrogenotrophic methanogens enriched from 11.3% to 80.2% relative abundance

# Graphical Abstract

To be filled later.

# Glossary

AD anaerobic digestion
ASV amplicon sequence variants
CCH4 concentration of CH4 in product gas
CSTR continuously stirred tank reactor
DNA deoxyribonucleic acid
fso fraction of electron donor for cell synthesis
GC-TCD gas chromatograph thermal conductivity detector
GC-FID gas chromatograph flame ionization detector
GHG greenhouse gas
HFR hollow-fiber reactor
MBfR membrane biofilm reactor
QCO2 volumetric CO2 delivery rate
QH2 volumetric H2 delivery rate
QPG flow rate of product gas
Qe sum of average liquid volume sampled and average effluent flow rate over time
Qv average liquid volume sampled from the reactor over time
SVR surface-to-volume ratio
TBR trickle bed reactor
RNG renewable natural gas
NTA nitrilotriacetic acid
ORP oxidation reduction potential
SA surface area
SCOD soluble chemical oxygen demand
SRT solids retention time
STP standard temperature and pressure
TSS total suspended solids
TSSv TSS measured within the reactor vessel
TSSe TSS measured just below the passive waste effluent
Vl total liquid volume of the system
Vr reactor volume
VSS volatile suspended solids
VFA volatile fatty acids
UV-vis ultraviolet-visible
k exponential decay constant for hydrogenotrophic methanogens
µ exponential growth rate constant
ρ Spearman’s rank correlation coefficient

# Introduction

Non-renewable fossil fuels such as natural gas, coal, and oil have empowered social advancements since the Industrial Revolution. However, anthropogenic activities, especially greenhouse gas (GHG) emissions, have caused the observed 1.1˚C rise in global average temperatures from 1850 to 2020 (IPCC, 2023). This has cascading negative effects across ecological, social, and economic systems, prompting efforts to curb global GHG emissions through development of renewable energy alternatives to fossil fuels. Adoption of renewable energy technologies additionally increase energy security and independence by diversifying options within the grid, particularly in regions where fossil fuels are not readily accessible (Cox et al., 2019; Fan et al., 2024; IEA, 2025).

Renewable natural gas (RNG), or CH4 derived from organic waste, presents a number of advantages for energy diversification. The bioprocesses producing RNG occur naturally in landfills and other organic waste streams, representing 26% of CH4 emissions in the United States (EPA, 2024). Capture and utilization of CH4 from degradation of organic waste can dually reduce GHG emissions and replace up to 25% of current global natural gas demand (IEA, 2025). Even with widespread adoption of wind, solar, and other renewables, natural gas will remain an important energy carrier in hard-to-decarbonize sectors such as industry and transportation. As a compliment to intermittent solar and wind power, RNG can be produced via power-to-gas technologies through biological conversion of waste CO2 and electrolytically produced green hydrogen (H2) to CH4, further intercepting and offsetting GHG emissions and providing energy storage (Lippert et al., 2025).

The most common process for RNG production is anaerobic digestion (AD), which yields biogas (≈60% CH4, ≈40% CO2). Biogas contains too much CO2 to be used directly as a drop-in fuel and must undergo upgrading processes to remove CO2. Biogas upgrading can be accomplished via biological, chemical, physical, or a combination of processes (i.e. pressure swing adsorption, water scrubbing) to remove CO2 and other contaminants (Aghel et al., 2022). The CO2 from these processes is discarded. As a biological upgrading technology, biomethanation can produce approximately 40% more CH4 than other processes. This is accomplished through the metabolism of methanogenic archaea, which converts CO2 and H2 into CH4 instead of simply removing the CO2. This can be done by either supplementing an anaerobic digester with H2 (i.e., in-situ) or by feeding biogas and H2 in a separate reactor (i.e., ex-situ) (Adnan et al., 2019). The H2 for this reaction is typically sourced from water electrolysis, ideally green H2 from solar or wind power to reduce the carbon footprint (Lippert et al., 2025).

H2 mass transfer limitations have been identified as a major technical challenge in ex-situ biomethanation systems, mainly due to its low solubility (Feickert Fenske et al., 2023b; Rittmann, 2015). Gaseous substrates H2 and CO2 need to enter the aqueous phase for microbial transformation to CH4 (Pauss et al., 1990; Phillips et al., 2017). To improve H2 mass transfer, several reactor configurations increase the gas-liquid interface via water droplets or gas bubbles. Trickle bed reactors (TBRs) consist of a columnar tank filled with high-surface-area biofilm carriers with a spray nozzle at the top delivering recirculating liquid. Substrate gases are either introduced from the base of the column or fed from the top, then diffuse into trickling liquid for delivery to biomass growing on plastic biofilm carriers. Several TBR pilot tests have been completed in full-scale biogas facilities, producing high quality biomethane (>97% CH4) at high production rates (Feickert Fenske et al., 2023a; Jønson et al., 2022; Sposob et al., 2021, p. 2022). However, TBRs are susceptible to operational challenges, including the formation of preferential flow paths and uneven wetting, sometimes leading to poor nutrient distribution and nonhomogeneous biofilm growth throughout the column (Tsapekos et al., 2021). The low relative volume of recirculating liquid in TBRs is furthermore prone to dilution from metabolic water production, which strongly affects buffering capacity and complicates pH control and nutrient delivery (Feickert Fenske et al., 2023a; Strübing et al., 2017a).

Continuously stirred tank reactors (CSTRs) are another common reactor configuration for biomethanation and use a gas diffuser and mechanical mixing to introduce substrate gases in a relatively large liquid volume. Mass transfer in CSTRs improves with increased agitation, gas recirculation, high pressure, and high temperature. However, maintaining these mass transfer-enhancing conditions can raise operational energy consumption and stress microbial cells. Literature furthermore indicates that CSTRs generally miss RNG quality targets (<90% CH4) (Fachal-Suárez et al., 2024). Other reactor configurations have been tested as well, including fixed bed reactors (FBRs), bubble columns, and plug flow reactors (PFRs). Each of these systems, however, contend with the same limitations of H2 diffusion via gas bubbles or water droplets.

One opportunity to increase the mass transfer of gases into the aqueous phase is the use of membrane technologies. Membranes provide a diffusive boundary layer between liquid and gas that maintains concentration gradients and promotes direct, bubbleless diffusion. Membranes can be employed for substrate delivery to suspended growth microorganisms, as in the case of a hollow fiber reactor (HFR) (Díaz et al., 2015a). Many studies have additionally explored the advantages of fostering biofilm development on the membrane surface, as in the case of membrane biofilm reactors (MBfRs) (Fachal-Suárez et al., 2024; Martin and Nerenberg, 2012). One study found the maximum mass transfer coefficients in clean water among five HFRs were up to 10 times higher than two TBRs and one CSTR. They attributed the improved performance to the high surface-to-volume ratio (SVR) in HFRs.

Membrane-biofilm reactors (MBfRs) combine the mass transfer advantages of membrane delivery of gaseous substrates with the high efficiency and process stability of biofilm reactors. In traditional biofilm reactors such as TBRs, biofilm attaches to the surface of an inert carrier. As biofilm thickness increases, microbial activity remains highest at the outside edge of the biofilm. Substrate availability decreases with depth into the biofilm, which can cause a dead zone at the attachment surface. Biofilm detachment via erosion or sloughing disproportionately impacts the surface layers of biofilms, meaning the active exterior surface layers are prone to loss and washout. In an MBfR, the inverse relationship exists; substrate concentrations are highest at the biofilm attachment surface, resulting in the development of the most active fraction of the biofilm at this location. Consequently, only the least active portion at the exterior of the biofilm detaches (Nerenberg, 2016).

A small number of small-scale feasibility studies on MBfRs for ex-situ biomethanation have been conducted under mesophilic (Miehle et al., 2021a; Pratofiorito et al., 2021) and thermophilic (Lin et al., 2026) conditions. Despite the theoretical advantages of membrane technologies, these studies reported low CH4 production rates relative to other configurations in literature. The authors of these studies described the low SVR of their systems as the limiting design parameter (Miehle et al., 2021a; Pratofiorito et al., 2021). Thus, the potential for process intensification (i.e., increased production of high-quality biomethane) in an MBfR with high SVR remains unexplored. In addition, the dynamics in the microbial community underlying function in ex-situ biomethanation MBfRs remains poorly resolved.

To fill these knowledge gaps, this study assesses the performance of a mesophilic, hollow-fiber MBfR with an eight to 21 times higher SVR than the above-described prior studies. The first objective was to test the capacity of the reactor to achieve stable production of high purity biomethane (≥95% CH₄, ≤5% H₂, and ≤2% CO₂). Following this demonstration, the next aim was to determine the MBfR’s maximum specific CH₄ production capacity while retaining quality >95% CH4. For the first time, this study also characterizes long term microbial community dynamics in a mesophilic MBfR for ex-situ biomethanation.

# Materials and Methods

## The reactor

The MBfR system consisted of a reactor vessel, sensor manifold, liquid/gas separator, gas delivery line, gas collection and cooling line, liquid recirculation, influent feed, and a passive waste line (Fig. 1). The volumetric parameters of this system included: total system volume (including headspace and peripherals) = 560 mL, total liquid volume = 410 mL, and reactor volume (including membranes) = 175 mL. Substrate gases, H2 and CO2, were delivered to both ends of a membrane bundle, creating dead-end delivery at mid-length. The bundle was hand-potted with 350 hydrophobic high-density polyethylene (HDPE) hollow fiber membranes with an outer diameter of 0.4 mm and a pore size of 0.2 µm (SFC Umwelttechnik GmbH, Salzburg, Austria). The membrane length was 330 mm, resulting in a total membrane surface area of 0.145 m2 and an SVR of 829 m2/m3, normalized to reactor vessel volume. The MBfR system was fabricated using stainless steel (SS), borosilicate glass, polyvinyl chloride (PVC), and polytetrafluoroethylene (PTFE) components. The gas-tightness and precision of volumetric gas recovery through the system was validated prior to inoculation.

### Figure 1

Substrate gases were delivered from pressurized tanks using mass flow controllers (MFCs) for H2 (MC-50SCCM-D-PCV03, Alicat Scientific, Tuscon, AZ, USA) and CO2 (32907-55, Cole Parmer, Vernon Hills, IL, USA). Effluent gas flow rates were measured using a gas counter with an upstream H2S scrubber (BlueVCount, BlueSens gas sensor GmbH, Herten, Germany), and product gas was collected in inert, multi-layer foil bags (Supelco Inc., Bellefonte, PA, USA) for time-averaged composite sampling. Continuous sensing and remote monitoring and control were accomplished using a commercial automation system adapted for the lab (Loxone Electronics GmbH, Kollerschlag, Austria). Sensing included temperature, pH, oxidation reduction potential (ORP), and aqueous CO2 in the liquid stream. A controlled, multichannel peristaltic pump delivered synthetic media and maintained pH at 7.0 ± 0.1 (Reglo ICC Pump, Cole-Parmer, Vernon Hills, IL, USA) using 1 M HCl and NaOH solutions. Temperature was maintained at 37 ± 1°C with a silicon heat tape and controller (TC4X Digital Temperature Controller in NEMA 4X Enclosure, Briskheat, Columbus, OH, USA).

Synthetic media was adapted from an existing recipe as follows (L-1): 0.3 g EDTA, 1 g NH4Cl, 0.6 g NaCl, 0.45 g K2HPO4, 0.2 g KH2PO4, 0.16 g MgCl2.H2O, 0.15 g CaCl2.2H2O, 15 mL 0.2M cysteine HCl, 1.6 mL of 50 g/L Na2S.9H2O, 10 mL of trace elements solution, 1 mL of vitamin solution. The trace elements solution contained (L-1): 1.5 g NTA, 0.8 g Fe(NH4)2(SO4)2, 0.2 g Na2SeO3, 0.1 g ZnSO4.7 H2O, 0.1 g NaWO4.2H2O, 0.01 g H3BO3, 0.1 g CoCl2.6H2O, 0.0068 g CuCl2.2H2O, 0.1 g NiCl2.6H2O, 0.1 g MnSO4.H2O, 0.1 g Na2MoO4.2H2O. The vitamin solution contained 5 mg/L each of biotin, folic acid, vitamin B-12, alpha-lipoic acid and 10 mg/L each of pyridoxine, thiamine, riboflavin, nicotinic acid, Ca-pantothenate, and p-aminobenzoic acid (Wolfe, 2011). At higher substrate loading rates, the sodium sulfide stock solution dosage was increased by 1-2 mL/L to meet increasing metabolic demands.

## Gas analysis

Product gas composition was measured using a gas chromatograph with a thermal conductivity detector (GC-TCD) equipped with a Haysep-D column calibrated for H2, CO2, and CH4 (SRI 8610C Multiple Gas Analyzer #5, SRI Instruments, Torrance, CA, USA). Calibration of analyte gases was conducted by manually diluting gases with the GC-TCD carrier gas, argon, in a 100 mL, gas-tight syringe in triplicate (CO2: 0.04%, 2%, 4%, 6%, 8%, 10%, 30%; CH4: 0.01%, 4%, 10%, 30%, 50%, 80%, 100%; H2: 0.01%, 1%, 2%, 4%, 6%, 8%, 10%, 30%). Calibrations used during initial operation of the reactor included a larger range of values for H2 and CO2. As MBfR performance improved, the calibration ranges were adjusted to represent more commonly occurring values.

## Liquid sampling and analysis

The membrane bundle was scoured twice per week (and prior to liquid and biomass sampling) by isolating the reactor from system periphery in a secondary loop. The recirculation rate was then increased from 0.2 L/min to approximately 3.5 L/min in both the forward and reverse direction for ten seconds.

Reactor influent media, reactor bulk liquid, and effluent waste liquid were analyzed for soluble chemical oxygen demand (sCOD) (TNT 821, Hach Company, Loveland, CO, USA), ammonia (TNT 832, Hach Company, Loveland, CO, USA), and total and volatile suspended solids (TSS/VSS) (Standard Method 2540D).  Volatile fatty acids (VFA), acetate, propionate, and butyrate, were measured by a gas chromatograph equipped with a flame ionization detector (GC-FID) (TRACETM 1310 GC Analyzer for Permanent Gases & Trace Impurities, Thermo-Fisher Scientific, Waltham, MA, USA).

The approach to calculate hydraulic retention time (HRT) and solids retention time (SRT) included the volume of samples removed from the reactor vessel and effluent line. This approach was applied for two reasons: 1) the relative volume of liquid sampled was high compared to total liquid volume, and 2) the TSS in the reactor vessel was higher than the TSS at the effluent. The following adapted equations were applied:

Hydraulic retention time HRT, days= VlQe+Qv#2

Solids retention time SRT, days= TSSvQeTSSe+QvTSSv#3

SRT was calculated considering planktonic growth and biofilm solids that were knocked loose from the membranes during mixing. Thus, the reported SRT is only a partial representation of the true system SRT, since the biofilm never fully detached.

## Specific CH4 production rate

All gas volume measurements and delivery setpoints were corrected to standard temperature and pressure (STP, temperature = 273.15 K, pressure = 1 atm). Ideal gas behavior was assumed for conversion between molar and volumetric values. For performance evaluations and comparison to literature, the reported CH4 production rate was normalized to two features: (i) the reactor volume (175 mL) according to Equation 4, to enable comparison to other reactor configurations and (ii) the membrane surface area (0.154 m2) according to Equation 5, to facilitate comparison to other membrane-based systems.

Volume based CH4 production rate m3CH4/m3d= CCH4QPGVr #(4)

Surface area based CH4 production rate NL CH4/m2d=CCH4QPGSA #(5)

Where CCH4 is the concentration of CH4 in the product gas (%), QPG is the flow rate of product gas, Vr is the reactor volume, and SA is the membrane surface area.

## Inoculum and microbial community analysis

The MBfR was inoculated using a methanogen-enriched anaerobic microbiome from an anaerobic laboratory system (fed H2 and CO2), which was initially inoculated with sieved (175 µm mesh) anaerobic digestate from the Stickney Water Reclamation Plant in the Metropolitan Water Reclamation District of Greater Chicago (Cicero, IL, USA).

Biomass samples were collected weekly from suspended growth biomass within the reactor vessel until the retention screen was removed (Day 170). After this, suspended growth biomass washed out of the reactor vessel, and biomass was sampled every 2-4 weeks by suctioning biofilm from the membrane via pipette instead. Duplicate DNA extractions for each sampling time point were performed using a FastDNA Soil Spin Extraction Kit (Thermo-Fisher Scientific, Waltham, MA, USA) and DNA quality and concentration were measured using UV-vis spectrophotometry (Eppendorf Biospec Fluorescence, Enfield, CT, USA).  The V4-V5 region of the 16S rRNA gene was amplified via polymerase chain reaction (PCR) using 515F/926R primers.  The Rush University Genomics and Microbiome Core Facility (Chicago, IL, USA) conducted barcoding via a second stage PCR, followed by 2 x 300 bp sequencing using Illumina MiSeq i100+ with IDT index with V3 chemistry. 16S rRNA gene amplicon sequence analyses followed previously described methods (Farmer et al., 2023). Briefly, raw paired-end reads were imported and processed in Qiime2 (2025.4-amplicon). Cutadapt was used to trim primers and DADA2 was used to trim and denoise reads to produce amplicon sequence variants (ASVs). Mafft and fasttree were used to generate phylogenetic trees and taxonomy was assigned to ASVs using the Qiime2 q2-feature-classifier plugin with a Naïve-Bayes classifier trained on the MIDAS v5.3 database13 (Dueholm et al., 2024). After taxonomic classification, outputs from QIIME2 were compiled in R v4.3.1 using phyloseq v1.44.0.

Relationships between microbial community members and operational conditions were conducted by generating Spearman’s rank correlation coefficients relating individual ASVs to one another. ASVs were additionally related to two operational parameters, H2 loadings and H2 breakthrough rates in the product gas. The correlations were tested for statistical significance via T-test.

## Experimental approach for the ex-situ biomethanation MBfR

The experimental approach first aimed to establish a stable performance baseline, defined as stable product gas quality meeting ≥95% CH4, ≤2% CO2, ≤5% H2 for at least 30 days without process upsets. This step enabled the identification of operational setpoints (e.g., stoichiometric feed ratio) that favor desired quality outcomes. The quality target was set according to DVGW G 260 Technical Rule (German Association for Gas and Water (DVGW), 2021). This target was selected to ensure compliance with a large number of natural gas distributors, however, it should be noted that many distributors accept lower fractions of CH4 with more H2 (Kapoor et al., 2019; Muñoz et al., 2015; Savickis et al., 2020; Thema et al., 2019). Subsequently, the maximum CH4 production capacity of the system was determined by systematically increasing substrate loading rates in a ramping phase. If gas quality met or exceeded 90% CH4 for three or more consecutive days, loading rate was increased by 5-10% relative to H2. This schedule was validated by comparing it to ramping regimes reported in literature (Strübing et al., 2017b).

# Results and Discussion

This study developed, optimized, and tested performance limits of an MBfR system for upgrading H2 and CO2 to CH4 over the course of 331 days. While the system yielded CH4 within days of startup, iterations on operational controls and influent media composition were conducted during the first 150 days before selecting the methodology described in Section 2. Accordingly, these results represent findings after the iterative period from day 150-331. Operational periods for discussion are designated as follows; adaptation to influent media (day 150-190), process stabilization (day 191-237), and ramping (day 238-331), which additionally encompasses performance peak (day 303-310) and exceedance (day 311-331).

## Gas quality, specific CH4 production rate, and substrate conversion efficiencies

The adaptation phase (day 150-190) represented a period of recovery from two process upsets caused by operational tests, followed by adaptation to the final selected influent media recipe. On day 145, a test media recipe, using 50 mL of 1 M HCl instead of NTA in the trace elements solution, severely inhibited methanogenesis. Gas quality reduced from 97.4% to 10.5% CH4 on days 146 and 149, respectively (data not shown). On day 150, the test recipe was replaced with the media described in Section 2.1. The subsequent recovery period was utilized to estimate the specific growth rate for the methanogens in the reactor (µ = 0.27-0.31/d, SI Section 1). The second operational test occurred on day 167. The substrate feed ratio (H2:CO2) was changed to 4.17:1 for a 26-hour period, which caused gas quality to decrease from 95.6% on day 167 to a trough of 46.9% CH4 on day 171.

After recovering from these tests (>95% CH4), the MBfR entered the stabilization phase (days 190-237), during which it demonstrated capacity to meet RNG quality targets (≥95% CH4, ≤5% H2, and ≤2% CO2) with an average of 95.7 ± 0.4% CH4, 1.0 ± 0.4% CO2, and 3.3 ± 0.9% H2 at 2.69 ± 0.15 m3CH4/m3d. Substrate conversion efficiencies were 88.8 ± 2.6% for H2 and 88.0 ± 2.3% for CO2, respectively, corresponding to a yield of 0.22 m3CH4/m3H2 (SI, Section 2).

During the ramping phase, the H2 loading rate was increased by 264% over 73 days while maintaining a substrate feed ratio of roughly 3.95:1 (Fig. 2b). The fraction of CH4 in product gas was observed to decrease in the days immediately after a loading increase but gradually recovered as biomass adapted. Despite the ramping, average quality remained as high as it was during stabilization from days 237-310, albeit with a greater fluctuation in CH4 and H2 concentrations (95.7 ± 1.7% CH4, 1.0 ± 0.4% CO2, and 3.2 ± 1.6% H2, days 190-275). The maximum observed quality during this stage was 98.8% CH4 at a production rate of 3.42 m3CH4/m3d (day 273). Gas quality dropped to its lowest point (75.6% CH4) after applying the highest H2 loading (39.6 NL H2/m2d) on day 310.

Several operational enhancements were employed after day 310 the exceedance phase to meet target gas quality, including:

- reducing the H2 loading to 39.2 NL H2/m2d without modifying the CO2 loading rate (3.95:1 to 3.90:1 feed ratio) to provide more CO2 for cell growth (day 314)

- increasing the recirculation rate to 0.4 L/min (day 314), 0.6 L/min (day 321), and then back to 0.4 L/min (Day 324) to improve H2 mass transfer (Orgill et al., 2013); the recirculation rate was dropped back down to 0.4 L/min to reduce strain on the membranes since 0.6 L/min resulted in transmembrane pressure exceeding manufacturer recommendations of <2 psi (SI, Section 3)

- dosing 1 mL of additional 50 g/L Na2S.9H2O solution in the influent media to meet increasing metabolic demand (Strübing et al., 2017b)

Despite these changes, average gas quality remained below target at 81.3 ± 3.8% CH4 at a production rate of 7.16 ± 0.44 m3CH4/m3d. This phase (day 310-331) was then determined to be performance exceedance. The next experimental step was to stabilize the system for 30 days at the prior loading rate (33.0 NL H2/m2d), where gas quality had reached 94% CH4 with potential for stable production of >95% CH4. However, a seal rupture in the membrane potting prevented a direct continuation of the presented work. Thus, the highest production rate approaching quality targets, or peak performance, was 6.18 ± 0.11 m3CH4/m3d (7.44 ± 0.21 NL CH4/m2d) at an average gas composition of 93.3 ± 1.0% CH4, 0.9 ± 0.7% CO2, 5.8 ± 0.5% H2 from day 303-310.

### Figure 2

## 3.2. Performance comparisons to literature

The observed maximum CH4 production rate in the MBfR was compared to other ex-situ biomethanation process alternatives in the literature. When normalizing to reactor volume, this MBfR’s maximum specific CH4 production rate (6.18 m3CH4/m3d) presented as more than twice as high as other mesophilic systems and met or exceeded the capacity of several thermophilic systems (Fig. 3a). Considering the higher CH4 production rates under thermophilic operation, this outcome suggests that increasing SVR can increase production capacity to a similar or even greater extent than increasing operating temperature.

While the volume normalized production rate for this system was significantly higher than the three compared MBfRs, it did not follow the 1:1 linear increases predicted in previous studies (Miehle et al., 2021a; Pratofiorito et al., 2021). To better understand the sources of discrepancy in expected vs. observed outcomes, production was additionally normalized to membrane surface area and compared to other membrane-based systems. The SVR in this system was eight, 21, and 35 times higher than MBfR 1 (Pratofiorito et al., 2021) and MBfR 2 (Miehle et al., 2021a), MBfR 3 (Lin et al., 2026), and the HFR (Díaz et al., 2015b), respectively (Fig. 3c). Conversely, the membrane-normalized production rate for this system was the lowest among the compared systems, at 7.5 NL CH4/m2d (Fig. 3b). This suggests that, while the system significantly outperformed comparative systems on a volumetric RNG production basis, additional gains in performance may be possible via optimization of membrane configuration or materials.

The selected membrane material may have limited the rate of H2 mass transfer. Observed transmembrane pressure (TMP) increased during the ramping phase under a constant recirculation rate prior to day 314 (SI, Section 3), indicating increasing resistance to diffusion. Porous membranes can become hydrated or clogged over time (Martin and Nerenberg, 2012), which may have occurred in this study. However, two of the other MBfR studies (reference) also used microporous membranes, so pore hydration does not fully describe the observed difference. In clean water tests, hydrophobic, non-porous membranes had the highest H2 mass transfer coefficients (Orgill et al., 2013b). Thus, the application of a nonporous membrane to our MBfR may enable even higher rates of CH4 production. but noted that performance varied greatly among modules depending on membrane materials and system hydraulics (Orgill et al., 2013).

The spatial arrangement of membrane fibers likely had a more significant effect on production capacity. The fibers in this system were closely packed (Fig. 1). Membrane fibers inside the bundle may have experienced lower shear rates than exterior fibers, leading to reduced mass transfer and nutrient deficiencies on the interior fibers. Thus, future research is warranted to evaluate different membrane materials, spatial arrangements, or operational strategies to ensure full utilization of membrane surface area.

Finally, a small fault in the seal of the membrane bundle, not detected in pre-experimental tests, appeared to have opened over the course of the experiment. This allowed occasional bubble breakthrough, bypassing the key operational advantage of membrane systems over other reactor configurations: bubbleless diffusion. Bubble breakthrough was first observed on day 182 and may have contributed to emergent hybrid operation (i.e., combined biofilm and suspended growth) toward the end of the experimental run (Fig. 4d). However, increased turbidity was also observed at high substrate loading rates in previous studies with and without bubble breakthrough (Miehle et al., 2021b; Pratofiorito et al., 2021).

It should be noted that hybrid MBfR operation (e.g. with both biofilm and suspended growth biomass) may be advantageous for overall performance. Competitive CH4 production can be accomplished in a membrane system via suspended growth alone  (Díaz et al., 2015a). Luo and Angelidaki (2013) found that the biofilm growing on hollow fiber membranes supplying H2 only contributed 22-36% of H2 consumption for an in-situ biomethanation CSTR, however, the application of this finding to an ex-situ system may be limited. The relative contribution of biofilm and suspended growth in this system was not directly quantified as part of this study and warrants future study. However, estimations using TSS/VSS measurements may provide insights. Biofilm detachment occurred intermittently in response to the mixing routine (leading to a high standard deviation in instantaneous SRT, 7.43 ± 11 days). On days 202, 209, and 293, the estimated fraction of VSS in the reactor, which had just detached from the biofilm into suspension, spiked to 70-91% of total system VSS (SI, Section 4). This indicates the biofilm generally represented ≥70% of total system biomass and likely a similar fraction of overall activity (as calculated using SI, Equation S5).

A notable outcome of this comparison is the performance of the HFR. Despite having the lowest SVR among the membrane-based systems, the HFR outperformed the compared MBfRs across production metrics excepting this study, normalized to reactor vessel volume (Fig. 3a, b). Temperature and operating conditions do not describe this difference, since MBfR 3 has a very similar operational setup to the HFR (e.g., similar SVR, temperature = 55˚C, gas recirculation) (Díaz et al., 2015a; Lin et al., 2026). The reason for the difference in performance between HFRs and MBfRs merits further investigation.

### Figure 3

## 3.3. Impact of the substrate feed ratio

The ratio of bioavailable H2:CO2 plays a critical role in selecting desired microbial functional groups and subsequent reactor products. The methanation reaction (Equation 1) describes a stoichiometric ratio of 4:1. However, this only accounts for the energy (e.g. catabolic) reaction; when accounting for reducing equivalents from H2 required for CO2 fixation and cell synthesis (fso = 0.08, neglecting biomass decay, see SI Section 5), the H2:CO2 ratio from biomethanation stoichiometry reduces to 3.7:1 (Rittmann and McCarty, 2020). Favored feed ratios in the literature range from 4:1 to as low as 3.6:1, likely as a reflection of the carbon uptake by diverse microbial communities rather than pure cultures (Fachal-Suárez et al., 2024). The feed ratio yielding stable, high-quality CH4 production for this MBfR was 3.95:1, suggesting the fraction of metabolism methanogens need for cell synthesis is lower in this configuration. This ratio supported high-quality gas production throughout most of the ramping phase. When gas quality significantly reduced during the exceedance phase (day 310-331), the feed ratio was lowered to 3.90:1 (day 314) to supply more CO2 for cell growth.

Changes in the H2:CO2 feed ratio significantly impacted reactor performance. As described in Section 3.1, on day 167, a 26-hour period with a 4.17:1 ratio caused a precipitous drop in gas quality from 95.6% to 46.9% CH4. The inline CO2 sensor and product gas composition (0% CO2) together indicated the system was CO2 limited. To amend this, the gas feed ratio was decreased to 3.71:1 on day 168 until the system recovered (i.e., gas quality approached 95% CH4) 18 days later. This recovery period yielded the highest observed VFA concentrations, including acetate (0.94 mM C), propionate (0.77 mM C) and butyrate (0.27 mM C). Generally, VFAs represented only a minor fraction of total COD in the reactor (SI, Section 6). Propionate and butyrate concentrations were otherwise low or non-detect throughout the experimental work, while acetate concentrations increased again during the final stages of the ramping phase when H2 loadings outpaced methanogenesis. The observed increases in VFAs could be due to excess H2. High H2 partial pressure favors acetogenesis over methanogenesis (Spirito et al., 2014) and promotes microbial chain elongation (Baleeiro et al., 2021; Calvo et al., 2021; Spirito et al., 2014). The summed relative abundance of homoacetogens in genera Acetoanaerobium and Acetobacterium was 1.0-1.5% leading up to this period, as compared to <0.8% for all subsequent biomass samples. However, taxa known to participate in chain elongation were not detected in significant abundances, so the sources of propionate and butyrate remain somewhat mysterious.

A 1-hour, incidental tenfold increase in the CO2 feed rate on day 296 rapidly reduced gas quality from 92.3% to 74.4% on day 297. Recovery from this event took 5 days. In this case, increased VFA production was also observed, but to a much lesser extent. While the effects perturbations in feed ratio were not explicit targets of this study, these observations suggest that the microbial community is more resilient to perturbations caused by excess CO2 than by excess H2.

## 3.4. Microbial community dynamics over the course of reactor operation

Partial 16S rRNA gene sequencing over the course of reactor operation revealed that the MBfR successfully enriched for a diverse community that included hydrogenotrophic methanogensand a number of non-methanogenic taxa (Fig. 4). We documented a total of 3,216 ASVs across all reactors sampling events. Several classified ASVs lacked detailed taxonomic resolution. To facilitate discussion, ASVs were assigned at the genus-level, or next defined taxonomic level if genus was unavailable, and assigned a number indicating an iterative count of the name to preserve uniqueness. A summary table of the iterative ID and corresponding ASV and Midas 5.3 taxonomic classification can be found in SI, Section 7.

We documented a progressive decline in overall alpha diversity in the MBfR microbiome. The Shannon Diversity Index decreased from 5.6 in the inoculum to an average of 2.4 ± 0.3 (days 149-300) and a low of 1.9 in the final biomass sample (Fig. 4). This decrease in alpha diversity corresponded to enrichment of known hydrogenotrophic methanogens affiliated with the family Methanobacteriaceae (Harirchi et al., 2022), from 11.3% in the inoculum to 80.2% on day 300 (Fig. 4). Despite the decrease in overall community diversity, the diversity of hydrogenotrophic methanogens increased. The two hydrogenotrophic methanogens among the top ten most abundant ASVs in the inoculum were replaced by five different ASVs in the mature community (Fig. 4), demonstrating functional redundancy over the course of the experimental run.  Among these, three were dominant. Methanobacterium.2 was the most abundant ASV during the stabilization phase on sampling dates 161, 182, and 189 (24-29%). Prior to stabilization and during the ramping phase, Methanobacterium.1 superseded Methanobacterium.2 as the most abundant ASV (52% by day 300). Methanobacteriaceae.1, also represented a major portion of the microbial community, but was never the most abundant.

The MBfR additionally supported a diverse flanking community. In an otherwise autotrophic system, putative heterotrophic fermenters made up the bulk of the non-methanogenic populations. Some fermenters were observed to have ephemerally high relative abundances. Enterobacteriaceae.1 emerged on day 149 (prior sampling dates not shown lacked this ASV) and peaked in the sample on day 161 (7.0%) correlating to the inhibitive test media introduced on day 145 which severely reduced gas quality and increased VFA concentrations in subsequent weeks (Fig. 2a, days 150-160). This is consistent with Enterobacteriaceae typically emerging during transitional periods or after process upsets (Kennedy et al., 2025). Burkholderiales.1, an ASV classified at the order level only, represented a major fraction of total relative abundance under stable substrate loadings (34% on day 182), but receded with increasing H2 loadings. In the absence of taxonomic resolution, the functional role of this ASV is difficult to predict. Other putative heterotrophic fermenters persisted through nearly all samples in low relative abundances, namely Lentimicrobium.1 (1.9-7.9%), Aminovibrio.1 (1.2-3.1%), and Petrimonas.1 (0.9-1.9%). The family Lentimicrobiaceae was also found in low abundance in a previous study in a hydrogenotrophic biofilm enriched with Methanoculleus, suggesting a syntrophic relationship between this Lentimicrobiaceae and hydrogenotrophic methanogens (Maegaard et al., 2019).

The observed fermenters may be saprophytic taxa that consume microbial decay products. Heterotrophic putative saprophytes have also been observed in other autotrophic biofilms (nitrifiers, etc.), with the fraction heterotrophs in biomass increasing with HRT (Nogueira et al., 2005). Considering the HRT of this system (3.17 ± 0.80 days), a reduction in HRT could reduce the fraction of heterotrophs in the microbial community. Additionally, some fermentative sulfur- and facultative sulfate-reducers emerged in low relative abundances, including the genera Desulfovibrio, and Mesotoga (Baffert et al., 2019; Grabowski et al., 2005; Harirchi et al., 2022; Maus et al., 2020; Nesbø et al., 2019).

The observed flanking community may have also contributed to biofilm formation. The MBfR biomass shifted from black suspended growth to a pale gray biofilm (SI, Section 8), consistent with high EPS production (Lin et al., 2026).  Methanogens have been observed to form thin biofilms in pure cultures, but the thick biofilm from this study more closely matches mixed culture biofilms (Bang et al., 2014).

### Figure 4

To further explore the relationship between methanogens and the flanking populations, patterns of co-associating taxa were elucidated with a correlational matrix of relative abundances (Fig. 6). Interestingly, Methanobacterium.2 had a strong (ρ = -0.78) and moderate (ρ = -0.59) negative correlation with Methanobacterium.1 and Methanobacteriaceae.1, respectively. This indicated potential interspecific competition between Methanobacterium.2 and the other two dominant methanogens within the mature community. Methanobacteriaceae.1 and Methanobacterium.1, conversely, had a very weak positive correlation (ρ = 0.10), indicating they did not compete with one another.

Based on correlation analysis, the flanking communities associated with the three key methanogenic ASVs appear to be unique and distinct from one another. In general, ASVs exhibiting positive correlations with Methanobacterium.2 were negatively or not strongly correlated with those of Methanobacterium.1 and Methanobacteriaceae.1, respectively, and vice versa with few exceptions (i.e., Proteiniphilum.1). Flanking community members appear to be unique to each methanogenic ASV, suggesting specificity in relationships between different methanogens and their flanking communities. Additionally, Methanobacterium.1 and Methanobaceriaceae.1 had more strongly negative correlations with flanking members, supporting the decrease in alpha diversity coupled with enrichment of Methanobacterium.1 and persistence of Methanobacteriaceae.1 over the course of the ramping phase.

### Figure 5

The observed divergence in flanking members may also have emerged as a result of increasing partial pressure of H2. To explore this, the relationship between molar H2 loadings, molar H2 breakthrough rates in the product gas, and trends in ASV relative abundances during different operational periods was elucidated in a correlational matrix (Fig. 6). The period from day 150-300, spanning all sequenced biomass samples, captures both stable operation and variable H2 loading rates with higher rates of H2 breakthrough to the product gas, and therefore, higher partial pressure of H2 in the liquid phase (up to 0.84 mg/L dissolved H2 according to Henry’s Law).

Using this approach, the only flanking members with statistically significant correlations to H2 were Thermovirga.1 (ρ = - 0.93, p < 0.05) and Desulfovibrio.1 (ρ = 0.89, p < 0.05). Some ASVs exhibited low or no correlations to H2 loadings or breakthrough. In these cases, the associations between flanking members and methanogens in Fig. 5 may not have been driven by H2, but were instead influenced by other operational conditions such as substrate feed ratio, HRT, or process upsets (i.e., Enterobacteriaceae.1). Among the methanogens, the only ASV demonstrating a significant and strong positive correlation with increased H2 loadings was Methanobacterium.1 (ρ = 0.79, p < 0.05), while Methanobacterium.2 had a strong negative correlation with increasing rates of H2 breakthrough (ρ = -0.93, p < 0.05). These results suggest Methanobacterium.1 may have a low H2 affinity. In a study on system response to H2 pulses, low-affinity methanogens were enriched in the presence of excess H2 (Agneessens et al., 2017). Conversely, under these conditions Methanobacterium.2 struggled to compete, while Methanobacteriaceae.1 was relatively unaffected.

### Figure 6

# Conclusions

This study developed and tested a novel H2-fed, high-SVR MBfR configuration for ex-situ biomethanation for 331 days. Results demonstrated robust and stable high quality (>95% CH4) RNG production at a rate of 2.69 m3CH4/m3d. Increasing substrate loading led to a progressive increase in production rate to 6.18 m3CH4/m3d at high quality (>93% CH4), along with a transition from biofilm to hybrid biomass. The maximum observed process performance of this mesophilic system was competitive with thermophilic process alternatives and exceeded all MBfRs presented in the literature to date. Interestingly, the membrane-normalized production rate (7.5 NL CH4/m2d) was lower than that of membrane-based systems in the literature, indicating potential for even higher production capacity with minor improvements to operations and design. Nevertheless, this system produced high quality biomethane at high rates under mesophilic conditions, demonstrating its suitability for RNG production.

Longitudinal microbial community analyses demonstrated strong enrichment of diverse hydrogenotrophic methanogens from a combined relative abundance of 11.3% in the inoculum to 80.2% in the final sample. Strong inverse correlations between Methanogenic ASVs within suggest interspecific competition the mature community exhibited interspecific competition and functional redundancy under various operating conditions. The methanogens were accompanied by a robust, low abundance (~20%) flanking community of non-methanogens, which changed in response to H2 loadings and exhibited unique associations with the key methanogenic ASVs.

Future work is warranted to investigate additional improvements to performance through minor changes to design elements, including 1) increasing slack in the membrane bundle to allow better exposure to shear forces on individual fibers and 2) evaluating performance with a nonporous membrane material. Additionally, considering the higher solubility of CO2 relative to H2, future work will also evaluate the effect of delivering H2 only in the membranes, with pre-dissolved HCO3- in the bulk liquid.

# Author Contributions

HN, TL, YL, LR, and GFW contributed to the conception and design of the study. HN, TL, PN, AF, DM, GB, and RAS contributed to data acquisition, analysis, and interpretation. HN, TL, GFW, and AF drafted the manuscript, and all authors contributed to critical revision for important intellectual content. All authors approved the final version of the manuscript and agree to be accountable for the work.

# Declaration of Competing Interest

The authors declare that they have no known competing financial interests or personal relationships that could have influenced the work reported in this paper.

# Acknowledgements

This work was possible due to funding through agreement DE-EE0009284 with the US Department of Energy under the direction of Beau Hoffman and Dr. Katie Davis. Additional thanks to Dr. Yingqian Xiong, Emily Kin, Dr. Zhen Jia, Dr. Yingfen Zhao, and Dr. Neil M. Schweitzer for their support and expertise in Northwestern labs.

# Data Availability

Data will be made available upon request.

# References

Adnan, Ong, Nomanbhay, Chew, Show, 2019. Technologies for Biogas Upgrading to Biomethane: A Review. Bioengineering 6, 92. https://doi.org/10.3390/bioengineering6040092

Aghel, B., Behaein, S., Wongwises, S., Shadloo, M.S., 2022. A review of recent progress in biogas upgrading: With emphasis on carbon capture. Biomass Bioenergy 160, 106422. https://doi.org/10.1016/j.biombioe.2022.106422

Cox, S., Beshilas, L., Hotchkiss, E., 2019. Renewable Energy to Support Energy Security. National Renewable Energy Laboratory.

Díaz, I., Pérez, C., Alfaro, N., Fdz-Polanco, F., 2015a. A feasibility study on the bioconversion of CO2 and H2 to biomethane by gas sparging through polymeric membranes. Bioresour. Technol. 185, 246–253. https://doi.org/10.1016/j.biortech.2015.02.114

Díaz, I., Pérez, C., Alfaro, N., Fdz-Polanco, F., 2015b. A feasibility study on the bioconversion of CO2 and H2 to biomethane by gas sparging through polymeric membranes. Bioresour. Technol. 185, 246–253. https://doi.org/10.1016/j.biortech.2015.02.114

Fachal-Suárez, M., Krishnan, S., Chaiprapat, S., González, D., Gabriel, D., 2024. An overview of biomethanation and the use of membrane technologies as a candidate to overcome H2 mass transfer limitations. Biotechnol. Adv. 77, 108465. https://doi.org/10.1016/j.biotechadv.2024.108465

Fan, Y., Chang, T., Ranjbar, O., 2024. Energy security and energy mix diversification nexus in the OECD countries. Econ. Anal. Policy 84, 2071–2085. https://doi.org/10.1016/j.eap.2024.11.012

Farmer, M., Sabba, F., Jia, Z., Dunlap, P., Barnard, J., Qin, C.D., Straka, L., Kozak, J.A., Downing, L., Wells, G., 2023. Confronting Assumptions of Phosphorus-Accumulating Organisms and Glycogen-Accumulating Organisms: Peaceful Coexistence in a Carbon-Limited Sidestream EBPR Demonstration. ACS EST Water 3, 2384–2394. https://doi.org/10.1021/acsestwater.3c00083

Feickert Fenske, C., Kirzeder, F., Strübing, D., Koch, K., 2023a. Biogas upgrading in a pilot-scale trickle bed reactor – Long-term biological methanation under real application conditions. Bioresour. Technol. 376, 128868. https://doi.org/10.1016/j.biortech.2023.128868

Feickert Fenske, C., Strübing, D., Koch, K., 2023b. Biological methanation in trickle bed reactors - a critical review. Bioresour. Technol. 385, 129383. https://doi.org/10.1016/j.biortech.2023.129383

German Association for Gas and Water (DVGW), 2021. Technical Rule - Standard DVGW G 260 (A) - Gas Quality.

IEA, 2025. Outlook for biogas and biomethane: A global geospatial assessment (Special Report), World Energy Outlook. International Energy Agency (IEA), France. https://doi.org/10.1787/040c8cd2-en

IPCC_AR6_SYR_LongerReport, n.d.

Jønson, B.D., Tsapekos, P., Tahir Ashraf, M., Jeppesen, M., Ejbye Schmidt, J., Bastidas-Oyanedel, J.-R., 2022. Pilot-scale study of biomethanation in biological trickle bed reactors converting impure CO2 from a Full-scale biogas plant. Bioresour. Technol. 365, 128160. https://doi.org/10.1016/j.biortech.2022.128160

Kapoor, R., Ghosh, P., Kumar, M., Vijay, V.K., 2019. Evaluation of biogas upgrading technologies and future perspectives: a review. Environ. Sci. Pollut. Res. 26, 11631–11661. https://doi.org/10.1007/s11356-019-04767-1

Lin, S.-H., Kuntke, P., De Smit, S.M., Hamelers, H.V.M., Gagliano, M.C., 2026. Membrane biofilm reactor (MBfR) enabling optimal H2 delivery and supporting minimal nutrient supplementation to hydrogenotrophic biofilms. Water Res. X 30, 100507. https://doi.org/10.1016/j.wroa.2026.100507

Lippert, T., Lin, Y.J., Nielsen, H.D., Wells, G.F., 2025. Neutral water electrolysis unifies green hydrogen and renewable natural gas production from biogas. Chem. Eng. J. 521, 166969. https://doi.org/10.1016/j.cej.2025.166969

Martin, K.J., Nerenberg, R., 2012. The membrane biofilm reactor (MBfR) for water and wastewater treatment: Principles, applications, and recent developments. Bioresour. Technol. 122, 83–94. https://doi.org/10.1016/j.biortech.2012.02.110

Miehle, M., Hackbarth, M., Gescher, J., Horn, H., Hille-Reichel, A., 2021a. Biological biogas upgrading in a membrane biofilm reactor with and without organic carbon source. Bioresour. Technol. 335, 125287. https://doi.org/10.1016/j.biortech.2021.125287

Miehle, M., Hackbarth, M., Gescher, J., Horn, H., Hille-Reichel, A., 2021b. Biological biogas upgrading in a membrane biofilm reactor with and without organic carbon source. Bioresour. Technol. 335, 125287. https://doi.org/10.1016/j.biortech.2021.125287

Muñoz, R., Meier, L., Diaz, I., Jeison, D., 2015. A review on the state-of-the-art of physical/chemical and biological technologies for biogas upgrading. Rev. Environ. Sci. Biotechnol. 14, 727–759. https://doi.org/10.1007/s11157-015-9379-1

Nerenberg, R., 2016. The membrane-biofilm reactor (MBfR) as a counter-diffusional biofilm process. Curr. Opin. Biotechnol. 38, 131–136. https://doi.org/10.1016/j.copbio.2016.01.015

Nogueira, R., Elenter, D., Brito, A., Melo, L.F., Wagner, M., Morgenroth, E., 2005. Evaluating heterotrophic growth in a nitrifying biofilm reactor using fluorescence in situ hybridization and mathematical modeling. Water Sci. Technol. 52, 135–141. https://doi.org/10.2166/wst.2005.0192

Orgill, J.J., Atiyeh, H.K., Devarapalli, M., Phillips, J.R., Lewis, R.S., Huhnke, R.L., 2013. A comparison of mass transfer coefficients between trickle-bed, hollow fiber membrane and stirred tank reactors. Bioresour. Technol. 133, 340–346. https://doi.org/10.1016/j.biortech.2013.01.124

Pauss, A., Andre, G., Perrier, M., Guiot, S.R., 1990. Liquid-to-Gas Mass Transfer in Anaerobic Processes: Inevitable Transfer Limitations of Methane and Hydrogen in the Biomethanation Process. Appl. Environ. Microbiol. 56, 1636–1644. https://doi.org/10.1128/aem.56.6.1636-1644.1990

Phillips, J., Huhnke, R., Atiyeh, H., 2017. Syngas Fermentation: A Microbial Conversion Process of Gaseous Substrates to Various Products. Fermentation 3, 28. https://doi.org/10.3390/fermentation3020028

Pratofiorito, G., Hackbarth, M., Mandel, C., Madlanga, S., West, S., Horn, H., Hille-Reichel, A., 2021. A membrane biofilm reactor for hydrogenotrophic methanation. Bioresour. Technol. 321, 124444. https://doi.org/10.1016/j.biortech.2020.124444

Rittmann, B.E., McCarty, P.L., 2020. Environmental Biotechnology: Principles and Applications, Second Edition. ed. McGraw Hill, New York.

Rittmann, S.K.-M.R., 2015. A Critical Assessment of Microbiological Biogas to Biomethane Upgrading Systems, in: Guebitz, G.M., Bauer, A., Bochmann, G., Gronauer, A., Weiss, S. (Eds.), Biogas Science and Technology, Advances in Biochemical Engineering/Biotechnology. Springer International Publishing, Cham, pp. 117–135. https://doi.org/10.1007/978-3-319-21993-6_5

Savickis, J., Zemite, L., Zeltins, N., Bode, I., Jansons, L., Dzelzitis, E., Koposovs, A., Selickis, A., Ansone, A., 2020. The Biomethane Injection into the Natural Gas Networks: The EU’s Gas Synergy Path. Latv. J. Phys. Tech. Sci. 57, 34–50. https://doi.org/10.2478/lpts-2020-0020

Sposob, M., Wahid, R., Fischer, K., 2021. Ex-situ biological CO2 methanation using trickle bed reactor: review and recent advances. Rev. Environ. Sci. Biotechnol. 20, 1087–1102. https://doi.org/10.1007/s11157-021-09589-7

Strübing, D., Huber, B., Lebuhn, M., Drewes, J.E., Koch, K., 2017a. High performance biological methanation in a thermophilic anaerobic trickle bed reactor. Bioresour. Technol. 245, 1176–1183. https://doi.org/10.1016/j.biortech.2017.08.088

Strübing, D., Huber, B., Lebuhn, M., Drewes, J.E., Koch, K., 2017b. High performance biological methanation in a thermophilic anaerobic trickle bed reactor. Bioresour. Technol. 245, 1176–1183. https://doi.org/10.1016/j.biortech.2017.08.088

Thema, M., Bauer, F., Sterner, M., 2019. Power-to-Gas: Electrolysis and methanation status review. Renew. Sustain. Energy Rev. 112, 775–787. https://doi.org/10.1016/j.rser.2019.06.030

Tsapekos, P., Treu, L., Campanaro, S., Centurion, V.B., Zhu, X., Peprah, M., Zhang, Z., Kougias, P.G., Angelidaki, I., 2021. Pilot-scale biomethanation in a trickle bed reactor: Process performance and microbiome functional reconstruction. Energy Convers. Manag. 244, 114491. https://doi.org/10.1016/j.enconman.2021.114491

us-ghg-inventory-2024-main-text_04-18-2024 (1), n.d.

Wolfe, R.S., 2011. Techniques for Cultivating Methanogens, in: Methods in Enzymology. Elsevier, pp. 1–22. https://doi.org/10.1016/B978-0-12-385112-3.00001-9